Schio, [DD/MM/’YY]

Messrs.

[COMPANY NAME]

[Address]

[Zip] [City], [Region]

[Country]

To the attn. of Mr./Ms. [Full Name]

OFFER NO...: [NNN/YY]

[INVENTORY]

- [INVENTORY DESCRIPTION] -

Max working width 130 cm

Operational modes:

- Printing roller in reverse (enamelling, printing, coating)

- Printing roller in synchro (back-coating, pattern & design printing) - optional

Machine composed of:

- main frame with solid steel side shoulders and stiffening crossbeams,

- adjustable levelling feet, minimum working height from floor 900 mm.

- main rubberized counter-roller, diameter 200 mm., rubber oils and solvents resistant, hardness 25 ShA, max eccentricity +/- 1/100 mm., driven,

- A/C gearmotor 0,37 Kw, inverter controlled, to drive the rubberized counter roller mounted on the counter-roller axle,

- no. 6 driven transport rubberized rollers, hardness 25 ShA, step 140 mm.

- A/C gearmotor 0,25 Kw., inverter controlled, with chain and sprocket gears to drive the transport rollers.

- air mandrel holder with pneumatic lifting and lowering by means of two pistons,

- A/C gearmotor 0,37 Kw., inverter controlled, to drive the air-mandrel. With toothed belt and pulleys,

- No. 1 doctor blade holder complete one composite made doctor blade, with manual adjustment of scraping pressure,

- stainless steel wastewater collection trays at the sides and under the counter-roller, with central drain,

- movable stainless steel washing tray for the engraved sleeve, the sleeve can be washed on the machine without the need to be taken out,

- motorized micrometric adjustment of coating thickness (clearance), with independent adjustment LH and RH sides and digital display on control panel; this setting is achieved by the vertical positioning of the engraved sleeve by electric jacks,

- mechanical parts for the electronic auto-positioning of the engraved printing roller (anti-drip device) in order to avoid excess delivery (overcoating) of enamel at the entry edge,

- electronic adjustment of working speed from 0,5 to 6 mt./min.

- electronic adjustment of engraved roller speed from 0,5 to 38 mt./min.

- Control panel with digital touch screen 8", fitted on the main cabinet, to check and set all the machine functions, fitted on RH side; list of available functions: line speed, printing roller speed, working thickness, anti-drip device settings, error messages and fault warnings,

- main electrical cabinet with electro-mechanical devices, inverters, router for remote assistance, protection IP55,

- set of machine accessories and instruction & maintenance manual,

Accessories included:

- No. 1 AIR MANDREL WW1300 for one engraved rubber sleeve

- No. 1 RUBBER POSITIVE SLEEVE WW1300 for reverse rotation.

Engraving code: 10C - for coating and enamelling

Total Price at EXW - loaded: 	GLASSTINTER 1300 	EUR 44.000,-

TECHNICAL SPECIFICATIONS

Nominal width				: 1.300 mm.

Max working width				: 1.300 mm.

Mechanical speed 				: 0-38 meters/min.

Pneumatic system feeding			: compressed air 6 bar

Pneumatic controls				: on the machine control panel

Electrical protection				: IP54 – CE norms

Voltage					: 400V 3ph+N / 50Hz

Installed electrical power 			: about 2,5 Kw

Color							: RAL 5010 - blue

NORMS

The machine will comply with below CE directives:

2006/42/CE (Machines directive)

2014/30/UE (EMC)

2014/35/UE (Low tension)

MAIN EXCLUSIONS

- Brickworks or any type of work for building, carpentry or other, for the installation of the machines.

- Transport and lifting means necessary to unload and install the machines,

- Compressed-air and all feeding system (tubes, valves, junctions, etc.) from the supply to the connection points of the machines,

- Electrical supply (wires, cable passes, terminals etc.) from the main el. line up to the main control panel,

- Supporting brackets for the pipelines and electrical ducts to the line main el. panel,

- Drain pipes from the machine connection points to the drain system,

- Air compressor, fumes depurator and filters,

- Raw materials,

- Anything else not clearly specified herein.

TERMS & CONDITIONS

Offer validity:	60 days

Payment: 	30% down payment by T/t transfer at the order

60% before loading

10% after start-up

Bank details: 	Unicredit Banca, Via Palladio #4, 36070 Trissino VI

ACCOUNT NO: 000105397238

SWIFT CODE: UNCRITM1N56

IBAN CODE: IT 11 T 02008 60810 000105397238

Erection & start-up:	Included in the price, at buyer’s charge hotel, food for one technician

Delivery terms: 	EXW - loaded

Delivery time: 	8 - 10 weeks from down payment receipt + transit time

Packing: 	Included, in wooden crate

Shipment: 	By truck

Guarantee: 	12 months

HS code: 	8451.8010.00

Looking forward to receiving your reply, we thank you for your kind attention and remain

yours faithfully,

RMD Srls

M. Bauce